Chp 17

Reaching Out to Remote Computers

Modems - Just a phone call away

Flow control   HW, SW

Buffering  program kernel HW

UUCP family

CU  -  Call Unix

Dials out to remote site and allows a remote terminal

login.  Uses the same system files as the other UUCP

commands.

cu -s38400 -ltty1A  1234567

~%put		~%take		~.

~!			~+lcmd		~%b

CT -  Call Terminal

Dials out to remote terminal/modem and presents a

login.

nohup ct -s38400 1234567

UUCP - Unix to Unix CoPy

Copies binary and text files to and from Unix systems.

A batch mode store and forward set of programs.

User access

Security features

Open directory structure

System configured access

Time tables

Configure for time and day dialout per system

uucico is the workhorse program

UUX - Unix to Unix eXecution

Allows a user to run a command on a remote system

and retrieve the results.

Used by mail (uucp), remote printing.

Spooler supports Dialup printers too.

uux    systemname!command

uux    systemname!command  !remote_file_arg

uux    systemA!command    systemB!filename

TCP/IP Networks

Addressing -  IP address

Class A		A.H.H.H

Class B		B.B.H.H

Class C		C.C.C.H

< 128 = A     128-191 = B    192-223 = C

Broadcast   Netmask

Subnet  and supernet

Ports -  0-1023

1024 and up

Network Login

Telnet - Allows a user to login to a remote host

across the network.			^]

telnet    host

telnet    IP addr

telnet    IP addr   port           telnet host  port

Telnet is a standard Unix Network command.

Requires telnetd to be running on remote host.

Rlogin - Allows a user to login to a remote host

across the network.			~.

rlogin  host

rlogin host -lusername

Berkley Unix command.    Uses .rhost file

if valid password.

host.equive

FTP -  File Transfer Program

Allows a user to login to a remote host and retrieve

files from or put files on the host.

Simple Command Set

ftp  host

bin			get			put			mget

mput		prompt		cd			lcd

chmod		dir			del			quit

Uses .netrc file to automate login process

Anonymous FTP

ftp	host

ftp/anonymous

abc@

logs you into a public directory

RCP -  Remote CoPy

Copies file to and from remote host

rcp  fileA   host:/dirpath/fileA

Uses .rhost and host.equive files

RCMD - Remote CoMmanD

Allows a user to run a command on a remote host.

rcmd    host	command

rcmd    host   -lusername  command

Uses .rhost file and host.equive

Ping - Send and ICMP ECHO_REQEST packet to

host.

ping   IP addr

ping host

A good way to see if the host is on the network.

Traceroute - Traces the route packets take to reach

the remote host.

traceroute host

Must be root to run.

Nslookup - Query Internet name server interactively

nslookup  host

nslookup ipaddr

NFS - Network FileSystem

Allows a user to access a file system as if it were

on the local host.

Server has /etc/exports

showmount -e host

Client Server - BIG WORDS little meaning !!!

X Windows -  A graphical unix interface.

Client server in the X model

Display

Screen

Mixed machine environments

X11 R6

Chapter 18   Some System Admin …

Page 470

start and stop    init and shutdown haltsys

set up new accounts  scoadmin acc

Establish defaults for user processes.

Organize groups and assign group ids.

set up links to other sites  (network?)

Maintain E-Mail  sendmail

Maintain printer service  lpstat scoadmin

Backups  tar and cpio

Tune the OS (kernel)   configure

Monitor performance  sar